Keep the unit without connecting any Utility or batteries, remove the PCB

Please use the multimeter to measure the diode voltage  the  main board element.

Please write down your measured values

Measure point:

- 1. Converter MOSFET& IGBT check(Main board)

| Parts | Attribute | Reference values | Failure status |
| --- | --- | --- | --- |
| ALL | Diode | SD: 0.556V DS: OL |  |

| Parts | Attribute | Reference values | Failure status |
| --- | --- | --- | --- |
| ALL | Diode | EC: 0.358V CE: OL |  |

- 2.BUCK  IGBT & Diode(Main board)

| Parts | Attribute | Reference values | Failure status |
| --- | --- | --- | --- |
| Q32/Q42 | Diode | CE: OL EC: 0.39V | Short or explosion |
| Q58 | Diode | + to -: 0.39V - to +: OL |  |

- 3.Inverter  IGBT (Main board)

| Parts | Attribute | Reference values | Failure status |
| --- | --- | --- | --- |
| ALL | Diode | EC: 0.378V CE: OL | Short or explosion |

- 4.BUS Soft and SPS (MOSFET &Diode)

| Parts | Attribute | Reference values | Failure status |
| --- | --- | --- | --- |
| Q36 | Diode | SD: 0.46V SG: 0.65V | Short or explosion |
| Q6 | Diode | SD: 0.46V SG: 0.29V | Short or explosion |
| D54 | Diode | + to -: 0.384V - to +: OL | Short or explosion |
| D65 | Diode | + to -: 0.448V - to +: OL | Short or explosion |
| Q1, Q2 | Diode | SD: 0.417V SG: 0.301V | Short or explosion |
| D5,D6 | Diode | + to -: 0.428V - to +: OL | Short or explosion |

- 7.MPPT Boost MOSFET &Diode  (MPPT board)

| Parts | Attribute | Reference values | Failure status |
| --- | --- | --- | --- |
| Q2, Q5 |  |  | Short or explosion |
|  | Diode | EC: 0.382V CE: OL |  |
| D1,D24 | Diode | + to -: 0.381V - to +: OL |  |

| Parts | Attribute | Reference values | Failure status |
| --- | --- | --- | --- |
| AC Fuse |  |  | Open |
|  | Resistor | 0R |  |